Specification

| ITEM | E-Book |
| --- | --- |
| Display | Technology: E Ink® “Electronic Paper” Screen Size: Approx. 5” Measured Diagonally Resolution: 800(H)×600(V) Gray Scale: Max. 8-Level Gray Scale |
| Media Format Supported | TEXT: TXT， html，PDF（To be updated） Image: JPG, PNG |
| Operating System | 2.6 linux kernel |
| Storage Media | SD Memory Cards |
| Support Language | Simplified Chinese |
|  | English and others to be updated |
| UI Language | English |
| Power | Battery Type: Rechargeable Lithium-Polymer Battery Life(Approx.): 3.7V, 1500mAh Recharging Time: Approx. 4 Hours with USB charging from powered Computer or Approx. 2 Hours when using AC wall charger AC Power: AC Charger requires 100-240V~, 50-60Hz, 0.6A |
| System Requirements(for download book data from usb) | Operating System: Windows® XP, 2000，vista Processor: 800MHz Intel® Celeron class processor or better RAM: 128MHz Minimum Hard Drive: 64MB Minimum available Hard Drive Space USB Port: USB 1.1 or faster USB port |
| Dimensions | Weight: Approx. 160 grams without cover Measurements: 108*152*12 mm (without cover) |
| Temperature | Operating temperature: 0 to 90 degrees F (0 to 50 degrees C) Storage temperature: -36 to 126 degrees F (-20 to 70 degrees C) |

Hardware Specification

| ITEM | Hardware Description |
| --- | --- |
| Processor | JZ4740 processor, clocked at 336MHz (NEW) |
| FLASH | 512MB FLASH(NEW) |
| RAM | 64MB SDRAM |
| Display | 5” PVI EPD |
| Input/output Ports and Expansion Interfaces | USB Device: USB mini B connector SD memory card slot |
| LED | Green color led: indicating work status Red color led: indicating charging status |
| Buttons | Power Switch: Power ON/OFF Volume Control: UP and DOWN Function Key: PageDown, PageUp, etc., |
| Power Consuming | Less than 300mA@3.3v at EPD refresh time(not include wireless module),Less than 1.5mA@3.3v at sleep time.It can show more than 7000 pages continuously with full charged battery. |