User Guide

PocketBook Pro 903

 
 
TABLE OF CONTENTS
CONTENTS 2
PRECAUTIONS 6
APPEARANCE AND CONTROLS POCKETBOOK PRO 903 1910
Front View: 10
Rear View: 11
Controls and Indicators 12
SPECIFICATIONS POCKETBOOK PRO 903 1914
GETTING STARTED POCKETBOOK PRO 903 1915
Step 1. Charging the Battery 15
Step 2. The inclusion of device 15
Step 3. Setting 16
Step 4. Customizing the Touch Pad 16
Step 5. Reading books 16
Step 6. Installing the USIM card for 3G Internet 16
User Guide 16
SOFTWARE FUNCTIONS 18
Main Menu 18
Latest Open Books 19
Main panel 20
Widgets 21
ELEMENTS OF MAIN PANEL 1922
Library 22
E-library «Bookland» 24
Notes 25
Favorite 25
Photo 27
Glossary 29
Appendix 30
Music 31
Search 1932
Settings 33
1. Device Information 33
2. Language / Language 33
3. Keyboard 33
4. Appearance 33
5. Shortcuts 34
6. Date / Time 36
7. When you switch to open 1936
8. Communications 36
9. Reading out the text 37
10. Service 37
REPLENISHMENT OF LIBRARIES AND READING OF ELECTRONIC BOOKS 1938
Listening to music 40
Set up a connection to Internet 41
Setting up a wireless connection via Bluetooth: 42
Configuring wireless WI-FI 1943
Configuring wireless 3G 1944
Work with the keyboard 45
Shortcuts 46
Search 1946
To 47
Voice 47
Notes 48
Bookmarks 48
On page 49
Settings 49
Glossary 49
Rotation of the book 50
Scale 50
Active content 51
POTENTIAL PROBLEMS AND THEIR ELIMINATION 1952
SERVICE CENTER 55
UPDATE SOFTWARE 56
COMPLIANCE WITH INTERNATIONAL STANDARDS 58
 
Thank you for purchasing
PocketBook Pro 903!

Before operating PocketBook Pro 903 be sure to read the manual. You'll learn how to ensure your safety and prolong the life of your device.
In case if you have not complied with the following precautions, or there are signs of problems, be sure to contact an authorized service center for testing the device. Failure to do so may result in failure of the device.


 
PRECAUTIONS

Do not expose to extreme high or low temperatures. Keep it away from direct sunlight. Do not expose the battery to strong heating or cooling. A hot battery might cause burns. Remember: the operating temperature of the device is from 0 ° C to 40 ° C.

Keep the device to open flames or smoke (a lit cigarette lighter, fire, etc.).

Do not expose the unit to liquid or moisture (read in the rain on the street at high humidity, near water, etc.). Do not let it liquid or moisture - this can cause a short circuit and damage the battery. If the equipment is exposed to liquid, immediately remove the battery pack and contact an authorized service center.

Do not place a long time in places with poor ventilation, such as a car or a tightly closed container.

Do not expose to strong magnetic fields.

Do not drop or try to bend the device. In cases of damage to the device, contact an authorized service center.

Do not disassemble or modify it. By the execution of works are allowed only employees authorized service center.

This device is not waterproof, it can not be used underwater. If the appliance falls into water immediately, contact an authorized service center.

Do not use the device if it has damaged the battery, charger (broken housing, is poor, broken power cord) or SD-card. When the battery is damaged (cracked housing, follows the electrolyte, has lost form, etc.) should be replaced immediately.

Do not replace the battery yourself. Using home-made or modified batteries may cause explosion and failure of the device.

Attention! Do not dispose of batteries in a fire - Explosive!

Attention! The screen is made on a glass substrate and is a fragile piece. Its surface is covered with a layer of flexible transparent material. Characteristic sign of mechanical damage to the screen area is that by changing the images are not of this site is updated, with visible cracks may not be. In the event of such symptoms, contact an authorized service center.

Avoid mechanical pressure on the screen more than 1H. Keep the screen objects that could scratch or damage it. To protect the screen, the transfer device in a protective cover or hardcover.

 
Do not use to clean the screen and body, organic and inorganic solvents such as benzene, etc. To remove dust from the surface of the device, use a soft cloth. To eliminate contamination, drip a few drops of distilled water. To get rid of stubborn stains, consult an authorized service center.

Do not expose the screen effects of electrostatic charge, artificial sources of electromagnetic radiation of ultraviolet frequencies.

Do not leave cables near sources of heat. When heating is possible deformation of the cable or melting of insulation, which can cause fire or electric shock.

Do not be disturbed connectors and the device keys. This can lead to a deterioration of their functioning and ultimately to failure.

Waste Electrical and Electronic Equipment (Applicable in the European Union and other European countries where there are separate collection systems)

Improper disposal of this product may cause a negative impact on the environment and human health. To prevent such consequences must fulfill special requirements for disposal of the device. The recycling of materials will help conserve natural resources. For more information about recycling device please contact your local city office, your household waste disposal service, shop where you purchased the device, or an authorized service center.

Unplug the wireless connection in places where the posters are available to "turn off two-way radio." Do not use a wireless connection on board and in the hospital, it may interfere with the aircraft or medical equipment. In addition, you can not use this device in areas where cellular communication is prohibited.

This unit receives and emits radio frequency energy and may cause interference to radio communications and other electronic equipment. When using personal medical devices (such as pacemakers and hearing aids), please consult with a physician or the manufacturer to find out whether they are protected from external RF signals.

Keep out of reach of children.

APPEARANCE AND CONTROLS POCKETBOOK PRO 903

In this section you'll find a common device of your PocketBook Pro 903, its function keys, and then can immediately begin working with him.

Front View:
 
 
Rear view:

 
 
Controls and indicators:

1. Key "On / Off" - turn on / off devices (long press);
2. "OK" - confirm, call the shortcut or shortcut menu;
3. The "up" - navigation button;
4. Key "Down" - navigation button;
5. The "left" - navigation button;
6. Key "Right" - navigation button;
7. Key "Return" - a return to the previous menu, closing the book. Long press - forced the closure of the program;
8. Button "Forward" - to the next page;
9. Button "Back" - go to the previous page;
10. Key "Menu" - a context menu;
11. Key "Home" - reaching the device's main menu;
12. Button "Reset" - restart the device;
13. Headphone jack - for connecting headphones with a jack 3,5 mm.
14. Speakers - 2 speakers on 0,5 are designed to play audio files;
15. Color indicator - is designed to display the status of the device when connected to a computer. Flashing orange - charging, green permanent - the battery is charged;
 
16. Built-in Micro-USB - for connecting devices to a computer or a charger with a cable Micro USB (charger will be optional);
17. Slot MicroSD - designed to connect external memory MicroSD. (MicroSD-card will be optional);
18. Battery cover;
19. Key "Volume" - an increase in the volume;
20. Key "Volume -" - decrease volume;
21. Key "Increase" - zoom;
22. Key "Reducing - - zoom out;
23. 3G On / Off. - On / Off button wireless 3G connection;
24. Stylus - used to control the device via the touch panel;
25. Touch Panel.
 
SPECIFICATIONS
POCKETBOOK PRO 903

Display 9,7''E-Ink ® Vizplex 1200x825, 155 dpi, black / white
Touchpad EMR (E-Writing, pen touch module)
Processor 533 MHz
Operative memory: 256 MB
Permanent: 2 GB
Linux ® Operating System
Connectivity Wi-Fi ® (b / g); Bluetooth ® 2.1; Micro USB (480 Mb / s)
MicroSD memory slot up to 32 GB
The stereo audio output for headphones 3,5 mm;
2 speakers of 0,5 W
There is an accelerometer
Book Format PDF (ADOBE DRM), EPUB (ADOBE DRM), PDF, EPUB, FB2, TXT, DJVU, RTF, HTML, CHM, DOC, TCR, FB2.ZIP
Image formats JPEG, BMP, PNG, TIFF
Audio formats MP3
Lithium-polymer battery, 3,7 B, 1530 mAh
Size 193x263x11, 5 mm
Weight: 530 g
 
GETTING STARTED POCKETBOOK PRO 903

Step 1. Battery charging
The first time the device must charge off from 8 to 12 hours. Charge the battery PocketBook Pro 903 in the following ways:
• from computer using USB-cable;
• from the network, using the USB-charging device (available optionally).
To charge the device for the first time, follow these steps.
1. Make sure the unit is turned off and not lit. If it is turned on, press and hold a few seconds, press the "On / Off."
2. Connect the device to your computer / network with the right cable.
3. The indicator lights amber when charging starts.
4. The indicator lights green or turns off when the battery is fully charged.
5. Now you can unplug the device and turn it on for first use.
If the device is turned on during charging, the screen PocketBook you are asked to choose an action - charging the battery or the connection for data transfer. Charging starts, only if you select the appropriate mode.

Step 2. Enabling Devices
To turn the unit on, press and hold the On / Off. "About two seconds. While turning on the screen saver appears. When the device is ready for use, the image will change the main menu, which presents
 
open last book, widgets, and the main features PocketBook Pro 903.

Step 3. Customize
 Follow the wizard's settings to configure the basic parameters before using the device. To select Use the arrow keys Up, Down, Right, Left and confirm pressing OK.

Step 4. Customizing the Touch Pad
Go to the Settings main menu, then Service ⇨ calibrate the touch panel. Sequentially press the stylus on the point of these programs on your screen.

Step 5. Reading books
At PocketBook Pro 903 have downloaded 1000 books of different genres and styles in different languages. To learn how to fill up your library, see "UPDATING THE LIBRARY and reading electronic books."

Step 6. Installing the USIM card for 3G internet
In PocketBook Pro 603 integrated 3G module, which enables you to connect to the Internet at any point where there coverage 3G. To install the USIM card is necessary to remove the back cover of the machine, loosen the mounting batteries, remove the battery itself, and then put into the opened slot USIM card.
To configure the 3G internet see Settings ⇨ Communication.

User Guide
User's guide is loaded into internal memory, it can drop at any moment and find a detailed description of the necessary functions you need.
Open the "Library". This displays a list of books available for reading. Open the book - the text immediately appears on the screen. To change the font size, press the "Up". Thus changes among 5 possible font sizes. To turn pages, use the arrow keys Left, Right, Back, Forward. "
Full user guide is also on the official website of the company PocketBook: http://pocketbook-int.com/support/pocketbook-Pro 903 /

Note: while the book opens, some features are unavailable. This is because there is a calculation the number of pages.
 
SOFTWARE FUNCTIONS
Main Menu
After switching on the device and download the screen displays the main menu.

 

Main menu - it is an element of software, which displays the basic functionality and control sections, allowing comfortable use device:

• The last open book;
• Main panel;
• Widgets.
To quickly access the main menu while using the device, press "Menu" on its front panel or in sequence, press "Reset".

Open last book

 

Displays a list of recently opened documents. The number of displayed documents depends on the type of layout, the main menu.

Main panel 

  

At this point the Main menu soderzhatcya core functionality and control sections of the device: 

1. "Library"; 
2. "Greatest Hits"; 
3. "Notes"; 
4. "Search"; 
5. "Applications"; 
6. "Music"; 
7. "Photographs and drawings; 
8. "Dictionary"; 
9. "Settings". 

Detailed information about each section of the main panel, see below. 
  
Widgets 

  

Widget - is the main menu items. Its main task - display current / interesting information and a quick transition to full information. Number and type of widgets may vary depending on the type of layout, the main menu. Manage this element can be made in the "Preferences" Under "Appearance". 
A standard set of widgets that can be displayed in the Main menu: 
• clock; 
• calendar; 
• Sudoku game; 
• vocabulary; 
• Music player; 
• calculator; 
• information about the user. 

  
ELEMENTS OF MAIN PANEL 
Library 

  

In the "Library" displays all the books, magazines and other documents are loaded into internal memory and MicroSD-card (if inserted). 

Main features of the section: 
 search and discovery of books and other documents in read mode; 
 content management interface through the context menu; 
 input into an electronic library «Bookland». 

Navigation: 
• press Up, Down, Right, Left - move through the list; 
• press "Forward", "Back" - a quick switch to previous / next page; 
• key "Return" - a return to the previous menu; 
• Press "OK" - confirm the selection. Pressing and holding down the "OK" for a few seconds causes the popup menu. 

Available menu commands in the "Library": 
  

• Open; 
• Information; 
• Mark as read / uncheck; 
• Add to Favorites; 
• File: 
o Rename; 
o Move to Folder; 
o Copy to SD-card; 
o Move to SD-card; 
o Delete; 
o Mark the group; 
• Type: 
o List; 
o Sketches; 
o Detailed; 
o The names on the list: 
 The name of the book; 
 The name of the file; 
o Filter: 
 No; 
 Only the internal memory; 
 Only the SD-card; 
o screen orientation; 
• Sort: 
o by name; 
o According to the author; 
o According to the series; 
o By date of establishment; 
• Find a book. 

  
E-library «Bookland» 

  

PocketBook Pro 903 allows you to browse and download books from the library e-bookshop «BookLand». The device gets access to library services through a wireless Internet connection (via Wi-Fi or Bluetooth). 
To switch to the electronic library in the "Library" then click «BookLand». 

Note: To access the library «BookLand» to establish a wireless connection to the Internet via WiFi or Bluetooth. More information about setting up and activating the connection, see the section "Customize". 

Key features: 
 login and user registration in the library; 
 Review and management of bookshelves and books to them; 
 access to new, popular books in-store or Free Library; 
 search for books in the catalog; 
 overview of popular authors; 
 overview of the ways to replenish the balance in the library «Bookland». 
  
Notes 

  

In this section, kept notes, you can create while reading through "shortcuts". Learn more about "shortcuts" on page 29. 

Favorites 

  

In this section are selected books or other documents that were added using the context menu under "Library". 

Main features of the section: 
 search and discovery of selected books or other documents in read mode; 
 content management interface using the context menu. 

Navigation: 
• press Up, Down, Right, Left - move through the list; 
• press "Forward", "Back" - a quick switch to previous / next page; 
  
• key "Return" - a return to the previous menu; 
• Press "OK" the exercise of choice. Pressing and holding down the "OK" for a few seconds causes the popup menu. 

Available menu commands in the "Favorites": 
• Open; 
• Information; 
• Mark as read / Unmark; 
• Delete. The "Delete" in this section removes the book just from your favorites list, the book itself is not deleted; 
• Create a category; 
• Move into the category; 
• Type: 
o List; 
o Sketches; 
o Detailed; 
o The names on the list: 
 The name of the book; 
 The name of the file; 
o Filter: 
 No; 
 Only the internal memory; 
 Only the SD-card; 
o screen orientation; 
• Sort: 
o by name; 
o According to the author; 
o According to the series; 
o By date of establishment; 
• Find a book. 

Photos 

  

In this section you can view photos and images in formats JPG, BMP, PNG and TIFF. 

Section menu consists of two points. 
1. Photo. Contains all the images stored in permanent memory device or on the MicroSD-card (if inserted). 
2. Screens. In this section you can view pictures taken with the device's screen. 

Main features of the section: 
 view photos and images; 
 content management interface using the context menu. 

Navigation: 
• press Up, Down, Right, Left - move through the list; 
• press "Forward", "Back" - a quick switch to previous / next page; 
• key "Return" - a return to the previous menu; 
• Press "OK" the exercise of choice. Pressing and holding down the "OK" for a few seconds causes the popup menu. 

  
Available menu commands in the "Pictures": 
• Open; 
• Information; 
• Delete; 
• Type: 
o List; 
o Sketches; 
o Detailed; 
o The names on the list: 
 The name of the book; 
 The name of the file; 
o Filter: 
 No; 
 Only the internal memory; 
 Only the SD-card; 
o screen orientation; 
• Sort: 
o by name; 
o On the date of creation. 

  
Dictionary 

  

This section provides a translation of the words with the help of installed dictionaries. To convert your favorite word, enter it using the onscreen keyboard, then the program will find its translation. 
For greater convenience, use full-screen mode - to do this, press and hold a few seconds, "OK" button. In this mode, you can browse the dictionary page by pressing "Right" and "Left". Press "OK" again to return to the input word. 
If your device has multiple dictionaries, you can switch between them by pressing the "Menu". 

Main features of the section: 
 Translation of words with the established dictionaries. 

Navigation: 
• press Up, Down, Right, Left »¬ - move on the icon letters on the keyboard; 
• Press "OK" - Select the letter; 
• press "Forward", "Back" - a quick switch to previous / next page; 
• key "Return" - a return to the previous menu. 

Applications 

  

This section contains games and additional software. 

Main features of the section: 
 launching additional programs and games. 

Navigation: 
• Keys: Up, Down - move by title applications; 
• Press "OK" - running applications; 
• key "Return" - a return to the previous menu; 

List of appendices: 
 Calculator - a simple calculator that allows you to perform four basic arithmetic functions, calculate the percentage and square root; 
 Watch - an application that eventually turns into a convenient PocketBook desktop digital clock; 
 Chess - The classic game of Chess. You can play with electronic opponent or another person; 
 Solitaire - Solitaire; 
 Sea Battle - The Game of Battleship; 
 Sudoku - a Japanese puzzle game "Sudoku"; 
 Snake - a game "Snake". 
Music 
  

This section of the "Main Menu" is used to listen to your favorite music in MP3. 

Main features of the section: 
 plays music in MP3 format; 
 Selecting the playback. 

Navigation: 
• press Up, Down, Right, Left "," OK "- control the music player; 
• key "Return" - a return to the previous menu. 

Note: the music player is the most energy-consuming device applications. When he runs in the bottom right corner of the screen icon notes. 
  
Search 

  

With this section, you can perform a quick search for books on your device. 

Main features of the section: 
 search for books / documents on the device PocketBook Pro 903. 

Navigation: 
• press Up, Down, Right, Left - move on the icon letters on the keyboard; 
• Press "OK" - Select the letter; 
• press "Forward", "Back" - a quick switch to previous / next page; 
• key "Return" - a return to the previous menu; 

Search. After logging in, this section appears on the screen keyboard, which you enter the name of the book or its author (see "Working with the keyboard). When finished turn on the "Search". To move the found book, use the keys "Down" and "Up", respectively. To exit the "Search", press "Menu" or "Return". 


Settings 

  

In this section you can configure the software, as well as view information about the device. 

Main features of the section: 
 device settings PocketBook Pro 903. 

Navigation: 
• press Up, Down, Right, Left - move the elements of the section; 
• Press "OK" - the entrance / selection; 
• key "Return" - a return to the previous menu. 

Key points in section 
1. Information about the device. Displays the model name, serial number, code of the hardware platform, firmware, full and free internal memory and microSD-memory card. 
2. Language / Language. Change language interface PocketBook Pro 903. 
3. Keyboard. Changing the keyboard layout. Changes take effect after you exit the "Settings". 
4. Appearance. Contains the following settings: 
• Interest. Change the theme of the software; 
Note: New themes can be downloaded from the website: http://www.pocketbook-int.com. 
  
• Font interface. At the bottom of the screen, a dialog box appears with a list of available fonts and preview the selected font. Select the font and exit the "Settings" to change to take effect. 
• Type bookshelf. Changing the look of the bookshelf. 
• The orientation of the screen. Choice Auto, portrait, landscape 90 °, 270 ° landscape or portrait orientation of the screen 180 °. 
• The screen saver at startup. Selecting wallpaper that will appear on the screen while powering the device. 
• Screen at shutdown. Selecting wallpaper that will appear on the screen during the shutdown device. 
• Display text. Select the desired display text (without smoothing, with smoothing or saturated). 
5. Shortcuts. To change modes, press the keystroke line with the right mouse button and then set the action from the popup list. 
5.1. For a "short press the On / Off", you can assign the following actions: 
• Keypad lock; 
• The main menu; 
• Exit; 
• The last open book; 
• The picture screen; 
• Viewing; 
• Open the player; 
• Off. 
5.2. For the other buttons and modes are available such action: 
• Quick Menu; 
  
• Transition: 
o Back; 
o Next; 
o Prev. Page; 
o ff. Page; 
o -10 pages; 
o +10 pages; 
o On page ...; 
o In the beginning; 
o In the end; 
o Prev. Section; 
o ff. Section; 
• Bookmarks: 
o Show the bookmark; 
o Set a bookmark; 
• Notes: 
o Write a note; 
o Save the page; 
o Open Notebook; 
• Links: 
o mode options; 
o Back on the link; 
• Contents; 
• Search; 
• Dictionary 
• About the Book; 
• Type: 
o Increase the font; 
o Reduce the font; 
o Turn; 
o Mode PDF; 
o Hide panel; 
  
• MP3 player: 
o Open player; 
o Pause / Play; 
• Volume +; 
• Volume -; 
• System: 
o The main menu; 
o Exit; 
6. Date / Time. In this section you can configure the following elements: 
• Set the current time and date; 
• Update mode clock: 
o When paging; 
o while navigating through (without shading); 
o time; 
o do not display the clock. 
• Lock the keypad: 
o off; 
o block after 5 minutes; 
o block in 10 minutes. 
• Mode switch off the device: 
o turn off; 
o turn off the device via 10/20/30/60 minutes. 
7. When you turn to open. At this point you can select which item to open when you turn on the device: 
• the main menu; 
• the last open book. 
8. Communication. You can adjust the settings for the device. This item consists of the following elements: 
• The name of the device; 
• Mode of access to the network; 
• Preferred connection; 
  
• Wireless connection Bluetooth; 
• Wireless WI-FI 
• Wireless 3G; 
• Connecting via USB port 
o PC connection; 
o charge; 
o ask on connection. 
• Mode USB-host 
o off; 
o on. 
9. Reading out the text. Settings voiceover text. You can: 
• Select the language and voice; 
• Select the speed of sound Text: 
o slow; 
o normal; 
o fast; 
• Choose the tone sound of the text: 
o 50%; 
o 75%; 
o 100%; 
o 125%; 
o 150%. 
10. Service. You can: 
• format the internal memory; 
• clean condition; 
• calibrate the touch panel; 
• calibrate the gyroscopic sensor; 
• delete the authorization ADE.

REPLENISHMENT OF LIBRARIES AND READING OF ELECTRONIC BOOKS

The device allows reading books in formats FB2, FB2.zip, TXT, PDF, RTF, HTML, PRC, CHM, DJVU, DOC, EPUB, TCR.

Preliminary steps.
1. Download a book in memory PocketBook Pro 903:
a) With the help of a computer. Connect your device to your computer using microUSB-cable. The internal device memory and MicroSD-card appear in Windows Explorer as the two new drives. Using Windows Explorer or other programs, copy books into the desired folder in the device memory or on SD-card.
b) Using the e-library «Bookland». Go to "Library" tab in the «Bookland». Be sure to connect to the Internet via available networks Wi-Fi, Bluetooth or 3G. In the window that appears you need to register and then you can download free and paid books, which will automatically fall into your library.

2. Open the book in the "Read".
To open a book, go to the Library and using the "Up, Down, Left, Right" and "OK", select the desired book. If it is one of the last open book, it can be accessed from the main menu. If you have already read the book, it opens to the page on which it was closed for the last time.

At the bottom of the screen shows the number of open pages, the total number of pages, current time and battery power. After the discovery of some formats are counting the number of pages. Before the end of the conversion to the bottom of the screen instead of the current page number and number of pages you will see the sign "...", and some functions will be temporarily unavailable.
 
LISTENING TO MUSIC

To listen to music on the device, follow these steps.
1. Download the audio files in MP3 format:
• Connect the device to the computer with microUSB-cable.
• Immediately after the connection Select the type of connection - "Charging the Battery" or "Mobile PC". Select the "Mobile PC"
• Internal memory device and memory card MicroSD (if available) will appear in Windows Explorer as a new drive. Copy files to Music folder.
2. Open the "Music" in the Main Menu.
3. Select from the list of song you want to listen first, and then press "OK".
4. Select the playback mode:
• one song;
• All songs by turns;
• All the songs in random order.
 
Get Connected to Internet

To connect to the Internet, follow these steps:
1. In the Main Menu go to "Settings", click "Communication";

2. Set the device name:
a) press "OK" on the appropriate line;
b) appears on-screen keyboard;
c) Enter the name that will be used to connect PocketBook Pro 903 with other devices;
d) Press "OK" to save the changes and "Cancel" to minimize on-screen keyboard.

3. Set the mode of access to the network. Click "OK" button on the bar, and select one of the following modes:
a) off;
b) is included;
d) request for permission.

4. Select your preferred compound. Pop-up list will list all available connections. Select the desired.

 
Setting up a wireless connection via Bluetooth:
a) Press the "OK" on the item «Bluetooth» and select the connection you want to configure. By default unconfigured compound called <Connecting № ...>.
b) Click the "Connection Name", will appear on-screen keyboard, enter the name that will be used to connect PocketBook Pro 903 with Bluetooth.
c) Specify the type of connection - an external modem (to enter the four PIN-code to connect via Bluetooth);
d) Set the access point, enter its name;
e) Enter the user name;
e) Enter your password;
g) Open the option "Disable if not active (minutes) and select from the drop-down list one of:
• 5;
• 10;
• 20;
• 30;
• 60.
h) Set up a proxy server. To do so, click "Proxy Settings" and using the onscreen keyboard, set the connection parameters:
• Proxy server;
• Proxy port;
• Name of the proxy user;
• Proxy password.
i) Check the connection. To do this, click Test connection. A message "Establishing connection". If the connection fails you will be able to see the event log.
 
Configuring wireless WI-FI
a) Open the list of wireless connections and select the appropriate. By default unconfigured compound called <wirelessly № ...>.
b) Click the "Connection Name", will appear on-screen keyboard, enter the name that will be used to connect PocketBook Pro 903 with Wi-Fi. Then press the "OK" to save changes and press "Cancel" to minimize on-screen keyboard.
c) Enter the network name (SSID) is similar to part b.
d) Specify the type of protection, selecting from the drop-down list one of:
• No;
• simple (WEP);
• WPA-Enterprise;
• WPA-PSK.
e) Open the item "Network Preferences, click" OK "button on the item" Configure IP-address ", and from the popup menu select:
• DHCP;
• manually.
e) Set the proxy server. To do so, click "Proxy Settings", and using the onscreen keyboard, set all proxy connections:
• Proxy server;
• Proxy port;
• Name of the proxy user;
• Proxy password.
g) Finally, check whether the connection. To do this, click Test connection. A message "Establishing connection". If the connection fails you will be able to see the event log.
 
Configuring wireless 3G

1. Click "OK" button to select "Wireless 3G» and select the connection you want to configure. By default unconfigured compound called <Connecting № ...>.
2. Click the "Connection Name", will appear on-screen keyboard, enter the name that will be used to connect PocketBook Pro 603 with 3G.
3. Specify the type of connection;
4. Set the access point, enter its name;
5. Enter the user name;
6. Enter your password;
7. Configure a proxy server. To do so, click "Configure Proxy" and using the onscreen keyboard, set the connection parameters.
8. Be sure to install the connection. To do this, click Test connection. A message "Establishing connection". If the connection fails you will be able to see the event log.
 
Work with the keyboard

 

To work with the keyboard using keys Up, Down, Left, Right and press «OK». To speed up the keyboard all the characters are divided into 5 zones. The first press performs a rapid transition in the middle of the zone selected, you can then select the desired character of the area. To switch the keyboard into Russian, Ukrainian and English languages, use the keys "RUS", "RBM" and «ENG», located on the bottom panel, or key language switch, located on the bottom right.
Long press the "left" is deleting the last entered character, press the "Up" - to delete all entered text.
 
Quick Menu

 

Shortcut menu invoked by clicking and holding down the "OK" in a book reading mode. You will see the following items:

Search

 
Allows you to quickly find a word in the book or its parts. After selecting the item from the shortcut menu appears on the screen keyboard, which you can enter the word (see "Working with the keyboard). After you enter the right word to select "Search", and in the lower right corner of the screen icon with a magnifying glass.
To move on the found words backward or forward, use the "left" and "Right", respectively. Found words will be highlighted. To exit the "Search", press the «OK». Search mode is not available for books in a format DJVU.

Contents
 
Go to the contents of the book. This feature is available to book in FB2, PDF, CHM, and only if the book is structured content.

Voice
 
This menu item launches the Quick Access feature TTS (Text To Speech), so you can listen to the text. That part of the text, which reads speaker, will be highlighted.
To change the volume, stop / resume play, or turn this feature off, press and hold "OK" in the listening mode of the text.
To access the main menu, press the "Menu", or sequentially, press "Reset".

Note: To change the "Language / Voice", "Speed of speech" and "The tone of sound Text, go to Settings» → «Reading out the text and set the necessary parameters.

Notes
 
Sometimes it is useful to keep the selection of the book in a separate notebook.
To create a new record in read-only mode, do the following:
1. Click a menu item shortcuts "Notes".
2. At the top of the screen bar appears to separate text. Press the "Up" and "Down, go to the top notes, and then click" OK "to assign an upper bound.
3. Similarly mark the end of the note.
4. Select notepad to save the new notes. A menu appears select: add a note to the existing notebook, or create a new one.
5. If you create a new notebook, the screen displays a keyboard. Enter a name for your new notebook / notes.
See notes can be made from the "Main Menu" (under "Notes"). Pressing the "OK" to view notes moves to a place in the book where it was made. In addition, notes can be viewed in the content of the book.

Bookmarks
 
In each book, you can create up to 30 bookmarks. Select "Bookmarks" from the shortcut menu. In the upper right corner of the page will appear the sign of a bookmark.
View and delete bookmarks you can from the contents of the book, which displays the actual content, bookmarks and notes.
Per page
 
This menu shortcut opens a window for selecting pages. Enter the number of the desired page and click "OK". You can quickly jump to the first or last page of the book using the "Forward" or "Back."

Settings
 
For books in the format of FB2, TXT, RTF, HTML, PRC, and CHM, and PDF mode «Reflow» You can find the most comfortable settings for reading - the kind and size of font, line spacing, page margins, as well as the file encoding for the books TXT format and HTML.

Dictionary
 
This feature allows you to translate the selected word while reading. Keys Up, Down, Right, Left "are used to select words on the page," OK "to switch between text window and the window dictionary.
In addition, for the translation of the word, you can use the virtual keyboard. The device pre-installed English-Russian dictionary N. Queen. Additional dictionaries are available at: http://www.pocketbook-int.com. Moreover, a converter is available, which will create a custom dictionary from a variety of free dictionaries available on the Internet.

Rotation of the book
 
This menu item shortcuts will set a convenient orientation of the screen. Rotation takes place in accordance with the selected icon. Selections are made by pressing the Up, Right, Down, Left. " "OK" includes automatic detection of image orientation.

Scale
 
In the books of the PDF and DJVU You can zoom the page in the shortcut menu.
Press the "Right", "Left" to select the desired display mode, the "up", "Down" set the scale.
The following modes:
1. "4 pages", "9 pages" - a preview mode, multiple pages per sheet;
2. "Normal" - one page on the screen with the ability to scale;
3. "Columns" - designed for reading books with columns on a page. To move a page in the "Columns" Use Up, Down, when the lower edge of the sheet will move to the next column;
4. "Layout» (Reflow) increases the readability of large format documents. In this mode, the text is formatted to the width of the device's screen. You can change the font size of the keys Up, Down. In the "arrangement" can not display tables and some pictures.
Active content

Active content includes sections of the book, made while reading the book bookmarks and notes (in the content they are in the wrong section, where they were created).
Use the "Up", "Down the transition between the points of the active content. Use the "Left", "Right" takes you to the previous / next page of active content. When you press "Menu" on the point of active content menu appears with these features:
• Cancellation;
• open;
• rename;
• edit text;
• remove;
• setting;
• expand the item.
In the active content by default, all items (sections) are collapsed (next to them is a sign ""). To expand / collapse item, press and hold "OK" or "Menu" and on the shortcut menu select:
• Expand the item;
• Minimize the point.

The transition to structured content (FB2, PDF, CHM)
To go to the contents of the book, then click the "Contents" from the shortcut menu.

Note: this feature is supported only for books in formats FB2, PDF, CHM with structured content.

 
POTENTIAL PROBLEMS AND THEIR ELIMINATION


Problem: The device does not turn a device include, but are only upload SPLASH
Possible cause: The battery is discharged.
Solution: Recharge the battery.

Problem: BOOK DO NOT OPEN - the message "possible that your book is damaged or PROTECTED" or just nothing happens.
Possible cause: The file is damaged books.
Remedy: If the book is loaded with the website, boot it up again. If the file is opened on a PC, but does not open on the device, save the workbook in a different format.

Problem: The book in HTML opens, but not display correctly.
For example, displays blank pages, some text is missing, etc.
Possible cause: HTML-files contain non-textual elements (frames, JAVA-script, flash elements).
Solution: save the page in a format TXT.

Problem: The book opens, but instead RUSSIAN letters on the screen Displays various characters, questions, etc.
Possible cause: incorrect encoding of the document.
Remedy: a book reading mode, choose "Settings" and change the encoding of the document.

Problem: The device does not respond by pressing a key.
Possible cause: in some cases (for example, when downloading the damaged or too bulky books) may "freeze" the device.
Correction: forcibly close the Hanging application by holding down the "Back" 1-2 seconds. If that fails, follow the forced reset the device by pressing the "Reset" on the rear panel.

PROBLEM: When connecting to a computer in Windows Explorer WINDOWS DOES NOT APPEAR storage device.
Possible cause: The device is in the battery.
Remedy: Check the USB mode under "Settings" main menu.
Set the "Mobile PC" or "Ask when you connect."
Possible cause: no signal on the line cord.
Remedy: Check for damage USB-cable, and securely it is inserted into the slots. Try to switch the USB-cable to another computer connector.

PROBLEM: NOT CURRENTLY FILLING, Settings, the current file position.
Possible cause: corrupted file system.
Remedy: Connect the device to your PC, then run disk scanning device. To do this, open My Computer, right click on the selected drive, click, select Properties - Tools - Check Disk ".
Possible Cause: Internal memory is full.
Remedy: Remove unneeded books, images and / or music.

 

PROBLEM: FAST discharge the battery.
Possible Cause: Auto shutdown is inactive.
Solution: From the main menu, open the section "Setting - Time." For the item "Disable the device by, choose the automatic shutdown (10, 20, 30 or 60 minutes).
Possible cause: running in the background music player. It is the most energy-intensive application device. On its activity can be found on the notes icon in the lower right corner.
Solution: Turn off music player.
 
SERVICE CENTER

Detailed information regarding the service can be found on the official site PocketBook International.

Link to website: http://www.pocketbook-int.com/support/.


 
UPDATE SOFTWARE

To update the software, follow these steps:
1. On the official website www.pocketbook-int.com open the section "Products";
2. Select PocketBook Pro 903;
3. Click on "Software and Documentation," which is on the right side of the screen;
4. In the "firmware", select the latest version of software for PocketBook Pro 903 and download it (for this you need to click the right mouse button on selected file ⇨ «Save Target As ...» ⇨ choose where to save the file ⇨ «Save");
"On this same page you can check out the new additions to your device, install new games, dictionaries, find the user manual.
5. Copy the firmware file «SWUPDATE.BIN» the card MicroSD (available optionally);
6. Remove the computer MicroSD-card with the firmware and insert it into PocketBook Pro 903;
7. Restart PocketBook Pro 903 with an inserted MicroSD-card by clicking on the Reset button on the bottom of the device;
8. In the boot device, press and hold the "Back" and "Next" until the message «Firmware update ...»;
9. Follow the instructions on the screen;
10. After all the settings, wait until you restart the device;
11. Firmware version can be checked in "Settings» ⇨ «Information about the device.
 
COMPLIANCE WITH INTERNATIONAL STANDARDS

01. The test equipment must be connected via USB-port to the appropriate source of limited power of a personal computer or laptop.
02. The test equipment must be connected via an adapter limited power source.
AC Power
Manufacturer
 FLEET
Model
 FCC050100U
Nominal Output Voltage
 5 VDC. current 1 A
 
The equipment is marked 0984 and allowed to operate in EU countries.

This marking indicates that the equipment complies with the EU Directive on radio communications and telecommunications terminal equipment 1999/5/EC, as well as the relevant sections of the following technical standards:

EN 300 328 - Electromagnetic compatibility and radio spectrum, broadband transmission systems, data transmission equipment in the range of 2.4 GHz ISM band technology with a distributed spectrum modulation; basic requirements of the European Committee for Electrotechnical Standardization, Art. 3.2 Directive on radio communications and telecommunications terminal equipment.

EN 301 489-17 - Electromagnetic compatibility and radio spectrum, EMC standard for radio equipment and services; Part 17. Specific conditions for wideband transmission systems and high-performance equipment for mobile local area networks of high power HIPERLAN.
EN 60950 - Information technology equipment. Safety.
EN 62311: 2008 - Assessment of electrical and electronic equipment with regard to restrictions of human exposure to electromagnetic fields (0 Hz - 300 GHz).
The label indicates the introduction of restrictions on use.
France: 2.4 GHz (metropolitan France):

In metropolitan France, the following frequency of public or private wireless networks:

Indoor: max. power (equivalent isotropic radiated power *) 100 mW in the range 2400-2483,5 MHz.

Outdoors: Max. power (equivalent isotropic radiated power *) 100 mW for the 2400-2454 MHz and max. power (equivalent isotropic radiated power *) 10 mW for the range of 2454-2483 MHz.




































"Contains Reader ® Mobile technology by Adobe Systems Incorporated"