PocketBook Pro Upgrade Manual

How to Upgrade Firmware of your PocketBook Pro

(models 602,902)

Important! Firmware upgrade is very important operation, that changes the controlling software of your electronic reader. In case it is performed incorrectly this software may be damaged and device might not function properly and has to be brought to the service center. Please, try to perform upgrade procedure as close to the steps in this manual as possible.

- Open site www.pocketbook-int.com. Select your country.

- Click the «Support» button on top menu. Select hardware model of your device in rulers. You can use the scrollbar.

- Download the Firmware to your PC.

- Unpack the content of the archive, file: SWUPDATE.BIN copy to your MicroSD-Card using a card reader or USB cable.

- Switch off PocketBook Pro by pressing button “On/Off”.

- Switch on PocketBook Pro with the inserted MicroSD-Card by pressing “On/Off”.

- Before the downloading of the device, keep pressed both buttons “Backward” and “Forward” until the message ‘Firmware update…’ is displayed on the screen.

- Follow on-screen instructions. While updating firmware the device will be reloaded for a few times.

- Wait for the message on the screen: “Please connect power cable”, then connect USB cable to PocketBook Pro.

- Wait for the message on the screen: “Please disconnect USB cable”, then disconnect USB cable.

- Wait for the message on the screen: “Please, Power On to continue”, then switch on the device, the firmware upgrade is completed.

Troubleshooting

| Error message | Reason | Correction method |
| --- | --- | --- |
| No upgrade image found | file SWUPDATE.BIN can not be found on the Micro SD card. Make sure the file is in the root directory of the card. | Try to format your Micro SD card (make a reserve copies of all its content prior to formatting) and copy the file to the root of the empty card. If this error persists, use another card. |
| Upgrade image is corrupted or Image is empty | – SWUPDATE.BIN file is damaged or there is no space available on the Micro SD card. | empty Delete some files form your card and copy SWUPDATE.BIN file to it one more time. Repeat the fownload from www.pocketbook-int.com. If this error persists, use another card. |
| File read error | Incorrect Micro SD card | – try using another Micro SD card |

Note: You can see your Firmware model in menu Configuration>About device